Table 11. Summary of implemented solutions and quantified benefits obtained after MFCA implementation.

| Quantity center | MFCA Solution implemented | Type of quantified benefit achieved | Investment (INR) | Resources saved in terms of Material/ Energy/ System cost | Annual savings (in INR) (1 USD = 69.55 INR) |
| --- | --- | --- | --- | --- | --- |
| 1. Raw material stockyard | To reduce the wall thickness variation up to 0.15 mm from the supplier end. | Approximately 19,500 tubes are issued per month. 9.75 INR/tube is saved by implementing the solution. | Requirement specified to the suppliers. (0 INR) | Material saving: Iron | 2,281,500 |
| 2. Lubrication | Stage 1: Cleaning of individual tube with pressurized water. Stage 2: Cleaning by pressurized air before putting the lot in an acidic bath. | ___ | ___ | ___ | ___ |
| 3.1 Pointing | Point length reduced up to 6 inches from 8 inches. | Approximately19,500 tubes are issued per month. 5.70 INR/tube is saved by implementing the solution. | 175,000 | Material saving: Iron | 1,333,800 |
| 3.2 Cold drawing | 1. Electric Proximity (inductive sensor) switch is introduced to avoid machine running in unload conditions. | Total 10 nos. of draw benches are present. Electricity is charged at the rate of 6.80 INR/ kWh. Approximately 450 kWh/month/draw bench is saved. | 12,000 | Energy saving: Electricity | 367,200 |
|  | 2. An inclined plate was provided with the oil tank and small stands were attached to the inclined plate so then the excessive oil is restored to the oil tank. | Approximately 25 kg of oil/month is prevented from being wasted by implementing the solution. Drawing oil is procured at 270 INR/kg. | 20,000 | Material saving: Oil | 81,000 |
| 3.3. Heat treatment and straightening | 1. Loading 2 Mt/hr at furnace instead of 1Mt/hr and is shut down when there is no load. | Cost is reduced from 2.21 INR/kg to 1.60 INR/kg. Loading in the furnace is approximately 6,00,000 kg/month. | ___ | Energy saving: Electricity, natural gas | 4,392,000 |
|  | 2. Introduced pneumatic cylinder due to which three persons are used for unloading instead of six. | By introducing pneumatic cylinder monthly expenditure of 3 workers is saved. The monthly wage expenditure of a worker is 13,000 INR. | 50,000 | Resource saving: Manpower | 468,000 |
|  | 3. For the elimination of the overheating problem, one extra cooling zone was introduced which increased material loading by 50%. | Treatment cost was reduced from 6 INR/kg to 2.75 INR/kg. Approximate loading is 2,00,000 kg/month. | 35,000 | Energy saving: Electricity | 7,800,000 |
|  | 4. Introduced C.L.I.T. (cleaning, lubrication, inspection, tightening) due to which breakdowns got reduced by 50%. | The utilization increased due to less breakdown. 15,000 kg of material was loaded more in the saved 50 hours. The loading/hr of the material is 300 kg. | Training given to concern people. (0 INR) | Energy saving: Electricity | 495,000 |
|  | 5. Tapping of waste heat (250 degrees) of effluent gas for pickling department instead of releasing in the air. | Approximately 5000 SCM PNG is saved by implementing the solution. PNG is procured at the rate of 38 INR/SCM. | 200,000 | Energy saving: Natural gas | 2,280,000 |
| 4.1.Cutting and deburring | 1. Electric proximity (inductive sensor) switch was introduced to stop machine when on unload condition. | Approximately 675 kWh/month/cutting machine is saved. [ 6.8 INR/ kWh * 675 units saved * 12 months * 2 machines] | 2,400 | Energy saving: Electricity | 110,160 |
|  | 2. One vertical pneumatic cylinder is added for proper gripping. | 1144 meters of tube length is saved/month by implementing this solution. 40 mm/ tube is saved. Rate of 1 meter tube is INR 48. | 1,000 | Material saving: Iron | 658,944 |
| 4.2.Rust prevention coating | Wastage of oil was recovered by using tapered plate attached with oil tank which resulted in productivity improvement and reduction in waiting time. | ___ | ___ | ___ | ___ |
| 5.Inspection and marking | Marking slats are used for at least 2 to 3 times instead of discarding it after single use. | The consumption of marking slate is reduced to 40 slates from 100 slates. The cost of one marking slate is INR 131. | ___ | Material saving: Wood and Cloth | 94,320 |
| 6.Packing and dispatch | Wooden crate thickness is reduced from 1 inch to 0.75 inches. | ___ | ___ | Material saving: Wood | 600,000 |